| 1 | Variable Transformation between Cartesian, Cylindrical and Spherical coordinate system to obtain the location of any object. |
| --- | --- |
| 2 | Determination and plotting of Electric field and Electric flux density in sphere full of charges with varying radius. |
| 3 | Determination and plotting of Force of attraction between two electric Charges and two parallel conductors separated by varying distance. |
| 4 | Analysis of Continuous Charge Distribution in electric field. To find the (i) line charge density, (ii) Surface charge density and (iii) Volume charge density for any specific (a) line, (b) surface or (c) volume. |
| 5 | Determination and plotting of Electric potential difference between any two points in free space medium separated by a distance R. |
| 6 | Determination and plotting of Capacitance of parallel plate capacitor in three different dielectric medium with specific area, dielectric thickness and various dielectric constant. |
| 7 | Determination and plotting of Capacitance of isolated and concentric sphere with different inner and outer radius and dielectric constant of the dielectric mediums. |
| 8 | Determination and plotting of Capacitance of Co-axial cable per unit length with different (i) inner radius, (ii) outer radius and (iii) relative permittivity of the dielectric medium. |

Exp 1.1

clc;

x=input('Enter the value of X=');

y=input('Enter the value of y=');

z=input('Enter the value of z=');

r=sqrt(x^2+y^2);

pi=atand(y/x);

disp('Cartesian to cylindrical coordinate system of p(r,pi,z)=',[r pi z]);

r=sqrt(x^2+y^2+z^2);

theta=acosd(z/r);

pi=atand(y/x);

disp('Cartesian to spherical coordinate system of s(r,theta,pi)=',[r theta pi]);

exp 1.2

clc;

r=input('Enter the value of r=');

pi=input('Enter the value of pi=');

z=input('Enter the value of z=');

x=r*cosd(pi);

y=r*sind(pi);

z=z;

disp('cylindrical to cartesian coordinate system of p(x,y,z)=',[x y z]);

r=sqrt(r^2+z^2);

pi=pi;

theta=acosd(z/r);

disp('cylindrical to spherical coordinate system of s(r,pi,theta)=',[r pi theta]);

xp 1.3

clc;

r=input('Enter the value of r=');

pi=input('Enter the value of pi=');

theta=input('Enter the value of theta=');

x=r*sind(theta)*cosd(pi);

y=r*sind(theta)*sind(pi);

z=r*cosd(theta);

disp('spherical to cartesian coordinate system of p(x,y,z)=',[x y z]);

p=r*sind(theta);

pi=pi;

z=r*cosd(pi);

disp('spherical to cylindrical coordinate system of s(p,pi,z)=',[p pi z]);

exp 4

clc;

clear;

q=input("Enter the charge value is:");

l=input("Enter the length value is:");

//line charge density

rl=(q/l);

disp('the line charge density is',[rl]);

//surface charge density

pi=3.14;

r=input("Enter the radius value is:");

rs=q/(4*pi*r^2);

disp('the surface charge density is',[rs]);

//volume charge density

rv=q/(1.33*pi*r^3);

disp('the volume charge density is',[rv]);

x = linspace(0, 10, 50);

y = rl*exp(-0.1 * x);

z = rs*exp(-0.1 * x);

v = rv*exp(-0.1 * x);

figure();

subplot(2, 2, 1);

plot2d3(x,y);

title('the line charge density is');

xlabel("length");

ylabel("rho l");

subplot(2, 2, 2);

plot2d3(x,z);

title('the surface charge density is');

xlabel("length");

ylabel("rho s");

subplot(2,2,3);

plot2d3(x,v);

title('the volume charge density is');

xlabel("length");

ylabel("rho v");

exp 4.1

// Parameters

clc;

L = 10; // Length of the wire (m)

Q = 50e-6; // Total charge on the wire (C)

// Calculate line charge density

lambda = Q / L; // Line charge density (C/m)

disp("Line charge density (lambda) = " + string(lambda) + " C/m");

// Distance range from the wire for electric field calculation

r = linspace(0.1, 5, 100); // Distance (m)

// Electric field calculation using Coulomb's Law for a line charge

ke = 8.99e9; // Coulomb's constant (N*m^2/C^2)

E = (ke * lambda) ./ r; // Electric field (N/C)

// Plot the electric field as a function of distance

plot(r, E, "r");

xlabel("Distance (m)");

ylabel("Electric Field (N/C)");

title("Electric Field from a Uniformly Charged Wire");

exp4.2

// Parameters

clc;

R = 30; // Radius of the disk (m)

Q = 5; // Total charge on the disk (C)

// Calculate surface charge density

A = %pi * R^2; // Area of the disk (m^2)

sigma = Q / A; // Surface charge density (C/m^2)

disp("Surface charge density (sigma) = " + string(sigma) + " C/m^2");

// Distance from the center of the disk to calculate electric field

z = linspace(0.1, 2, 100); // Distance (m)

// Electric field on the axis of the disk

ke = 8.99e9; // Coulomb's constant (N*m^2/C^2)

E_disk = (ke * sigma * R^2) ./ (2 * (z.^2 + R^2).^(3/2)); // Electric field (N/C)

// Plot the electric field as a function of distance

plot(z, E_disk, "b");

xlabel("Distance (m)");

ylabel("Electric Field (N/C)");

title("Electric Field from a Uniformly Charged Disk");

exp4.3

// Parameters

clc;

R = 1.6; // Radius of the sphere (m)

Q = 2.3; // Total charge (C)

// Calculate volume charge density

V = (4/3) * %pi * R^3; // Volume of the sphere (m^3)

rho = Q / V; // Volume charge density (C/m^3)

disp("Volume charge density (rho) = " + string(rho) + " C/m^3");

// Distance from the center of the sphere

r = linspace(0.1, 3, 100); // Distance (m)

// Electric field inside and outside the sphere (using Gauss's Law)

ke = 8.99e9; // Coulomb's constant (N*m^2/C^2)

E_sphere_inside = (ke * rho * r) / 3; // Electric field inside the sphere (N/C)

E_sphere_outside = (ke * Q) ./ r.^2; // Electric field outside the sphere (N/C)

// Plot the electric field inside and outside the sphere

plot(r, E_sphere_inside, "g", r, E_sphere_outside, "b");

xlabel("Distance (m)");

ylabel("Electric Field (N/C)");

title("Electric Field from a Uniformly Charged Sphere");

legend("Inside", "Outside");

exp5

clc;

clear;

q=input("Enter the q value:");

e=8.854*10^-12;

pi=3.14;

ax=input('Enter the value of ax=');

ay=input('Enter the value of ay=');

r1=sqrt(ax^2+ay^2);

v1=q/(4*pi*e*r1);

disp('The value of v1',[v1]);

bx=input('Enter the value of bx=');

by=input('Enter the value of by=');

r2=sqrt(bx^2+by^2);

v2=q/(4*pi*e*r2);

disp('The value of v2',[v2]);

r=r1-r2;

v=v1-v2;

disp('The electric potential difference is',[v]);

r1 = linspace(0, 10, 50);

r2= linspace(0, 10, 50);

r= linspace (0, 10, 50);

y = v1*exp(-0.1 * r1);

z = v2*exp(-0.1 * r2);

x = v*exp(-0.1 * r);

figure();

subplot(2, 2, 1);

plot2d3(r1,y);

xlabel("radius 1");

ylabel("voltage 1");

title('The potential difference v1');

subplot(2, 2, 2);

plot2d3(r2,z);

xlabel("radius 2");

ylabel("voltage 2");

title('The potential difference v2');

subplot(2, 2, 3);

plot2d3(r,x);

xlabel("radius");

ylabel("voltage difference");

title('The potential difference v');

exp 5.1

// Constants

epsilon0 = 8.854e-12;            // Permittivity of free space

k = 1 / (4 * %pi * epsilon0);    // Coulomb constant

// User Inputs

r1 = 0.8;     // meters

r2 = 1.6;     // meters

q  = 15*10^-6;     // Coulombs

// Potential Difference: ΔV = k*q*(1/r1 – 1/r2)

deltaV = k * q * (1/r1 - 1/r2);

// Display result

disp("Potential Difference ΔV (in volts) = " + string(deltaV));

// Optional: Plot if you want r2 range

r2_range = linspace(r1+1, 50, 100);

deltaV_plot = k*q*(1/r1 - 1 ./ r2_range);

clf();

plot(r2_range, deltaV_plot, "b");

xlabel("r2 (m)");

ylabel("ΔV (V)");

title("Potential Difference for Given r1 = 15 m, q = 12 C");

exp5.2

// Constants

epsilon0 = 8.854e-12;            // Permittivity of free space

k = 1 / (4 * %pi * epsilon0);    // Coulomb constant

// Case 2 Inputs

q_case2 = 6e-6;      // 6 × 10^-6 C

r1_case2 = 0.8;      // meters

r2_case2 = 1.2;      // meters

// Potential Difference ΔV = k*q*(1/r1 – 1/r2)

deltaV_case2 = k * q_case2 * (1/r1_case2 - 1/r2_case2);

// Display result

disp("Potential Difference ΔV (in volts) = " + string(deltaV_case2));

// Optional plot (r2 range)

r2_range = linspace(r1_case2 + 0.1, 20, 100);

deltaV_plot = k * q_case2 * (1/r1_case2 - 1 ./ r2_range);

clf();

subplot(3,1,1);

plot(r2_range, deltaV_plot, "b");

xlabel("r2 (m)");

ylabel("Delta V (V)");

title("Case 2: q = 6×10^{-6} C, r1 = 2.8 m");

exp5.3

// Constants

epsilon0 = 8.854e-12;            // Permittivity of free space

k = 1 / (4 * %pi * epsilon0);    // Coulomb constant

// Case 3 Inputs

q_case3 = -6e-6;     // -6 × 10^-6 C (negative charge)

r1_case3 = 3.3;      // meters

r2_case3 = 1.2;      // meters

// Potential difference ΔV = k*q*(1/r1 – 1/r2)

deltaV_case3 = k * q_case3 * (1/r1_case3 - 1/r2_case3);

// Display result

disp("Potential Difference ΔV (in volts) = " + string(deltaV_case3));

// Optional: Plot with small variation around given values

r1_range = linspace(3.0, 3.6, 100);

r2_range = linspace(1.0, 1.4, 100);

// Compute ΔV across range

deltaV_plot = k * q_case3 * (1 ./ r1_range - 1 ./ r2_range);

clf();

plot(r1_range, deltaV_plot, "g");

xlabel("r1 (m)");

ylabel("Delta V (V)");

title("Case 3: q = -6×10^{-6} C, r1 = 3.3 m, r2 = 1.2 m");

exp 6

clc;

clear;

a1=input("Enter the a1 value is:");

d=input("Enter the d value is:");

e=8.854*10^-12;

c=e*a1/d;

disp(c);

d = linspace(0, 10, 50);

y = c * exp(-0.1 * d);

plot2d3(d,y);

xlabel("d");

ylabel("capacitance");

title("Parallel plate capacity");

a2=input("Enter the a2 value is:");

d1=input("Enter the d1 value is:");

d2=input("Enter the d2 value is:");

d3=input("Enter the d3 value is:");

er1=input("Enter the er1 value is:");

er2=input("Enter the er2 value is:");

er3=input("Enter the er13 value is:");

c1=e*a2/((d1/er1)+(d2/er2)+(d3/er3));

disp(c1);

exp6.1

// Case 1: AIR

a1=input("Enter the a1 value is:");

T=input("Enter the d value is:");

epsilon_0 = 8.854e-12;         // Permittivity of free space

area = 5;                   // 5 cm^2 = 5 × 10^-4 m^2

thickness =1;              // 1 cm = 1 × 10^-2 m

epsilon_r_air = linspace(1.0, 1.5, 100);   // Relative permittivity range

// Capacitance formula: C = ε_r * ε_0 * A / d

capacitance_air = (epsilon_r_air .* epsilon_0 .* area) ./ thickness;

disp(capacitance_air);

// Plotting in pF

clf();

plot(epsilon_r_air, capacitance_air * 1e12, 'b');

xlabel("Dielectric Constant (\epsilon_r)");

ylabel("Capacitance (pF)");

title("Capacitance of Parallel Plate Capacitor with Air");

xtitle("Capacitance vs Dielectric Constant for Air");

grid();

exp6.2

a1=input("Enter the a1 value is:");

T=input("Enter the d value is:");

epsilon_0 = 8.854e-12;

area = 3;

thickness = 5;

epsilon_r_glass = linspace(4.0, 7.0, 100);

capacitance_glass = (epsilon_r_glass .* epsilon_0 .* area) ./ thickness;

disp(capacitance_glass);

plot(epsilon_r_glass, capacitance_glass * 1e12, 'g');

xlabel("Dielectric Constant (\epsilon_r)");

ylabel("Capacitance (pF)");

title("Capacitance of Parallel Plate Capacitor with Glass");

xtitle("Capacitance vs Dielectric Constant for Glass");

grid();

exp6.3

// Case 3: Positive charge, both r1 and r2 vary

a1=input("Enter the a1 value is:");

T=input("Enter the d value is:");

r1_case3 = linspace(0.5, 5, 100); // meters

r2_case3 = linspace(5, 10, 100); // meters

deltaV_case3 = k * q * (1 ./ r1_case3 - 1 ./ r2_case3);

// Plotting

clf();

subplot(3, 1, 3);

plot(r1_case3, deltaV_case3, "g");

xlabel("r1 (m)");

ylabel("Delta V (V)");

title("Case 3: Positive Charge, Both r1 and r2 Vary");

exp7

clc;

clear;

pi=3.14;

e=8.854*10^-12;

r1=input("Enter the r1 value is:");

r2=input("Enter the r2 value is:");

ci=4*pi*e*r2;

disp('Capacitence of isolated',[ci]);

r2=linspace(0, 10, 50);

x=ci*exp(-0.1*r2);

plot2d3(r2,x);

xlabel("raduis 2");

ylabel("capacitence");

title("isolated capcitence is");

co=(4*pi*e*(r1*r2))/(r2-r1);

disp('Capacitence of consolated',[co]);

exp7.1

clc; clear;

// Constants

epsilon_0 = 8.854e-12;

R1 = 3; Rm = 0.6; R2 = 2;

// Dielectric constants

epsilon_r1 = 1:0.5:5;

epsilon_r2 = 1:0.5:5;

// Capacitance calculation

C = zeros(length(epsilon_r1), length(epsilon_r2));

for i = 1:length(epsilon_r1)

for j = 1:length(epsilon_r2)

C1 = 4 * %pi * epsilon_0 * epsilon_r1(i) * (R1 * Rm) / (Rm - R1);

C2 = 4 * %pi * epsilon_0 * epsilon_r2(j) * (Rm * R2) / (R2 - Rm);

C(i, j) = 1 / (1 / C1 + 1 / C2);

end

end

// Plotting

clf;

surf(epsilon_r1, epsilon_r2, C');

xlabel("Dielectric Constant (Region 1)");

ylabel("Dielectric Constant (Region 2)");

zlabel("Capacitance (F)");

title("Capacitance of Spherical Capacitor");

colorbar();

exp8.1

clc; clear;

// ---- USER INPUTS ----

r1 = input("Enter inner radius r1 (in meters): ");

r2 = input("Enter outer radius r2 (in meters): ");

rho = input("Enter dielectric constant εr (rho): ");

// ---- CONSTANT ----

epsilon0 = 8.854e-12;   // F/m

// ---- CHECK CONDITIONS ----

if r1 >= r2 then

disp("Error: r1 must be LESS than r2");

abort;

end

// ---- CALCULATE RATIO ----

ratio = r2 / r1;

// ---- CAPACITANCE PER UNIT LENGTH FORMULA ----

// C = (2π * ε0 * εr) / ln(r2/r1)

C = (2 * %pi * epsilon0 * rho) / log(r2 / r1);

// ---- DISPLAY OUTPUT ----

mprintf("\n----- RESULTS -----\n");

mprintf("Inner radius (r1)     = %.6f m\n", r1);

mprintf("Outer radius (r2)     = %.6f m\n", r2);

mprintf("Dielectric constant ρ = %.3f\n", rho);

mprintf("Radius ratio (r2/r1)  = %.4f\n", ratio);

mprintf("Capacitance C         = %.6e F/m\n", C);

// ---- OPTIONAL PLOT ----

r1_range = linspace(r1*0.5, r2*0.9, 300);

C_plot = (2 * %pi * epsilon0 * rho) ./ log(r2 ./ r1_range);

clf;

plot(r1_range*1000, C_plot*1e12, 'b-', 'LineWidth', 2);

xlabel("Inner Radius (mm)");

ylabel("Capacitance (pF/m)");

title("Capacitance per Unit Length vs Inner Radius");

grid on;

legend("C vs r1");

exp8.2

clc; clear;

// ---- USER INPUTS ----

r1 = input("Enter inner radius r1 (in meters): ");

r2 = input("Enter outer radius r2 (in meters): ");

rho = input("Enter dielectric constant εr (rho): ");

// ---- CONSTANT ----

epsilon0 = 8.854e-12;   // F/m

// ---- CHECK CONDITIONS ----

if r1 >= r2 then

disp("Error: r1 must be LESS than r2");

abort;

end

// ---- CALCULATE RATIO ----

ratio = r2 / r1;

// ---- CAPACITANCE PER UNIT LENGTH FORMULA ----

// C = (2π * ε0 * εr) / ln(r2/r1)

C = (2 * %pi * epsilon0 * rho) / log(r2 / r1);

// ---- DISPLAY OUTPUT ----

mprintf("\n----- RESULTS -----\n");

mprintf("Inner radius (r1)     = %.6f m\n", r1);

mprintf("Outer radius (r2)     = %.6f m\n", r2);

mprintf("Dielectric constant ρ = %.3f\n", rho);

mprintf("Radius ratio (r2/r1)  = %.4f\n", ratio);

mprintf("Capacitance C         = %.6e F/m\n", C);

// ---- OPTIONAL PLOT ----

r1_range = linspace(r1*0.5, r2*0.9, 300);

C_plot = (2 * %pi * epsilon0 * rho) ./ log(r2 ./ r1_range);

clf;

plot(r1_range*1000, C_plot*1e12, 'b-', 'LineWidth', 2);

xlabel("Inner Radius (mm)");

ylabel("Capacitance (pF/m)");

title("Capacitance per Unit Length vs Inner Radius");

grid on;

legend("C vs r1");

exp8.3

clc; clear;

// ---- USER INPUTS ----

r1 = input("Enter inner radius r1 (in meters): ");

r2 = input("Enter outer radius r2 (in meters): ");

rho = input("Enter dielectric constant εr (rho): ");

// ---- CONSTANT ----

epsilon0 = 8.854e-12;   // F/m

// ---- CHECK CONDITIONS ----

if r1 >= r2 then

disp("Error: r1 must be LESS than r2");

abort;

end

// ---- CALCULATE RATIO ----

ratio = r2 / r1;

// ---- CAPACITANCE PER UNIT LENGTH FORMULA ----

// C = (2π * ε0 * εr) / ln(r2/r1)

C = (2 * %pi * epsilon0 * rho) / log(r2 / r1);

// ---- DISPLAY OUTPUT ----

mprintf("\n----- RESULTS -----\n");

mprintf("Inner radius (r1)     = %.6f m\n", r1);

mprintf("Outer radius (r2)     = %.6f m\n", r2);

mprintf("Dielectric constant ρ = %.3f\n", rho);

mprintf("Radius ratio (r2/r1)  = %.4f\n", ratio);

mprintf("Capacitance C         = %.6e F/m\n", C);

// ---- OPTIONAL PLOT ----

r1_range = linspace(r1*0.5, r2*0.9, 300);

C_plot = (2 * %pi * epsilon0 * rho) ./ log(r2 ./ r1_range);

clf;

plot(r1_range*1000, C_plot*1e12, 'b-', 'LineWidth', 2);

xlabel("Inner Radius (mm)");

ylabel("Capacitance (pF/m)");

title("Capacitance per Unit Length vs Inner Radius");

grid on;

legend("C vs r1");